# PAPER_46_HARDWARE_BOM.md

**Document ID:** PAPER-46

**Version:** 2.3 (Authoritative Hardware Draft)

**Status:** Frozen Scope — Hardware, Electrical, Mechanical, Power, and Procurement Only

**Architecture Baseline:** Unified Qualcomm-Centric Hardware Architecture (QRB5165)

---

## 1.0 Architectural Hardware Baseline

The ReflexOS hardware platform is consolidated on a unified Qualcomm QRB5165 (Thundercomm TurboX RB5) compute architecture[cite: 1]. All secondary microcontroller tiers (including ESP32-S3 class co-processors) are eliminated from the hardware baseline[cite: 1]. Sensor ingestion buses, low-voltage GPIO lines, audio serial interfaces, and power management telemetry interface directly with the Qualcomm QRB5165 System-on-Module (SoM) and carrier board assembly via dedicated voltage level translators and standardized positive-locking connectors[cite: 1].

All non-hardware specifications are excluded from this document in accordance with repository scope rules:

* Board Support Package (BSP) device tree mappings and kernel driver configurations: **Moved to Paper 40**[cite: 1]

* Audio DSP pipeline scheduling and HRTF calibration behaviors: **Moved to Paper 42**[cite: 1]

* Mesh network topology and consensus protocols: **Moved to Paper 44**[cite: 1]

* Dynamic thermal throttling daemons and runtime power arbitration: **Moved to Paper 45**[cite: 1]

* Blind-first interface design, physical control human factors, haptic patterns, speech workflows, and accessibility validation criteria: **Moved to Paper 47**

---

## 2.0 Master Hardware Bill of Materials (BOM)

Table 2.1 defines the complete, canonical hardware Bill of Materials for the ReflexOS platform. Every component is referenced by its exact manufacturer part number across all subsequent sections.

### Table 2.1: Master Hardware Bill of Materials

| Item | Subsystem | Manufacturer | Exact Part Number | Description / Hardware Function | Voltage Domain | Interface / Bus | Qty (Rec. Build) | Unit Price (USD) | Ext. Price (USD) |

| :--- | :--- | :--- | :--- | :--- | :--- | :--- | :--- | :--- | :--- |

| 1 | Compute | Qualcomm / Thundercomm | **RB5-DK-QRB5165** | TurboX RB5 SOM & Carrier Board (QRB5165 ARM64 + Hexagon DSP)[cite: 1] | 12.0V DC Input (1.8V I/O) | PCIe, SPI, I2C, I2S, UART, USB 3.1 | 1 | $495.00 | $495.00 |

| 2 | Storage | Western Digital | **SDPTN-001T-1122** | WD Black SN770M 1TB M.2 2230 PCIe Gen4 NVMe SSD[cite: 1] | 3.3V DC | M.2 Key M (PCIe Gen3 x2) | 1 | $109.00 | $109.00 |

| 3 | Security / TPM | Infineon Technologies | **SLB9670VQ2.0** | OPTIGA TPM 2.0 Hardware Root-of-Trust IC[cite: 1] | 1.8V / 3.3V DC | SPI (up to 43 MHz) | 1 | $4.50 | $4.50 |

| 4 | Radar | Texas Instruments | **AWR1843BOOST** | 77 GHz to 79 GHz Automotive mmWave Radar Sensor Module[cite: 1] | 5.0V DC Input (3.3V I/O) | SPI + Dual UART | 1 | $299.00 | $299.00 |

| 5 | UWB | Qorvo | **DWM3000EVB** | IEEE 802.15.4z Ultra-Wideband Transceiver Module[cite: 1] | 3.3V DC | SPI (up to 38 MHz) + IRQ GPIO | 1 | $24.00 | $24.00 |

| 6 | IMU | Bosch Sensortec | **BMI088** | 6-Axis Vibration-Robust Inertial Measurement Unit[cite: 1] | 1.8V / 3.3V DC | SPI (up to 10 MHz) + INT GPIO | 1 | $8.50 | $8.50 |

| 7 | Audio Codec | Realtek | **ALC5682I-CG** | Low-Power High-Performance I2S/SoundWire Audio Codec[cite: 1] | 1.8V Digital / 3.3V Analog | I2S + I2C Control | 1 | $3.80 | $3.80 |

| 8 | Audio Amplifier | Texas Instruments | **TAS2563YBGEVM** | 6.1W Digital Input Smart Class-D Audio Amplifier (Stereo Pair) | 5.0V Boost / 1.8V I/O | I2S + I2C Control | 2 | $4.20 | $8.40 |

| 9 | Microphones | InvenSense / TDK | **ICS-43434** | 24-Bit Digital I2S MEMS Omnidirectional Microphone (NRND SPH0645 Replacement) | 1.8V / 3.3V DC | I2S (24-bit) | 2 | $3.50 | $7.00 |

| 10 | Speakers | Visaton | **SC 5.9 - 8 OHM** | 50 mm Miniature