Technical specification defining the exact component models, dimensions, pin layouts, and mechanical envelopes for the silicon, audio, security, storage, and power subsystems of reflexOS, establishing precise CAD boundaries.

Paper 52: Exact Hardware BOM & Mechanical Dimensions Specification

1. Microphones

•

Component Model: Knowles SPH0645LM4H-B (I2S Digital MEMS Microphone)  2 (Stereo Beamforming Array).

•

Exact Dimensions: 3.50 mm  2.65 mm  0.98 mm per surface-mount package.

•

Acoustic Port Clearance: Requires a 0.50 mm diameter circular sound hole through the enclosure wall, aligned directly with the bottom acoustic port of the MEMS package with an internal 0.2 mm acoustic sealing gasket.

•

Mounting & Envelope: Soldered directly to the Audio Brick PCB. Service Envelope: 10 mm  10 mm clearance zone above the package for acoustic baffling.

2. Speakers

•

Component Model: Visaton SC 5.9 (50 mm Miniature Full-Range Speaker Driver, 8 Ohm)  2 for spatial audio and voice output.

•

Exact Dimensions: Circular frame diameter: 50.0 mm; Overall mounting depth: 21.5 mm; Magnet diameter: 32.0 mm.

•

Mounting Hole Locations: Four mounting ears spaced on a 58.0 mm diagonal pitch using M2.5 fasteners.

•

Acoustic Enclosure Envelope: Requires a sealed, tuned back-cavity volume of minimum  molded directly into the Audio Brick housing to prevent acoustic short-circuiting and deliver clean bass response down to 200 Hz.

3. Audio Amplifier

•

Component Model: Texas Instruments TPA3116D2 (30W Class-D Stereo Power Amplifier IC mounted on a breakout/driver carrier board).

•

Exact Dimensions (Carrier PCB): 45.0 mm  35.0 mm  12.0 mm (including heatsink).

•

Mounting Hole Locations: Four M2 holes at 38.0 mm  28.0 mm centers.

•

Thermal & Service Envelope: Requires a 5.0 mm clearance above the integrated aluminum heatsink fins for convective airflow within the Audio Brick enclosure.

4. Trusted Platform Module (TPM 2.0)

•

Component Model: Infineon SLB 9670 (TPM 2.0 SPI Security Chip, PG-DSO-28 package).

•

Exact Dimensions (Breakout / Module PCB): 18.0 mm  16.0 mm  3.5 mm.

•

Interface & Mounting: Soldered directly to the Compute Brick motherboard via a dedicated SPI header; zero user-serviceable mechanical extraction (secured permanently under the secure enclave housing shield).

5. Storage (NVMe SSD Boot Storage)

•

Component Model: Western Digital WD Black SN770M NVMe SSD (M.2 2230 Form Factor).

•

Exact Dimensions: 30.0 mm  22.0 mm  2.38 mm.

•

Mounting: Secured via a single M2 standoff at the 2230 retention notch, lying flat parallel to the Compute Brick PCB.

•

Thermal Envelope: Topped with a low-profile 2.0 mm copper thermal fin spreader; Installation Envelope requires 35 mm  25 mm footprint clearance.

6. Wi-Fi / Bluetooth Module (Standalone Peripheral Card)

•

Component Model: Intel Wi-Fi 6E AX210 (M.2 2230 NGFF Module).

•

Exact Dimensions: 30.0 mm  22.0 mm  2.4 mm.

•

Interface & Antennas: M.2 Key E slot connection; dual IPEX/U.FL coaxial antenna connectors routed to chassis-mounted panel antennas.

•

Service Envelope: 35 mm  25 mm  8 mm clearance to allow coaxial cable snap-on attachment without bending cables beyond their 10.0 mm minimum bend radius.

7. Battery Management IC (BMIC) & Power Subsystem

•

Component Model: Texas Instruments bq25713 (I2C-controlled Buck-Boost Battery Charge Controller IC) paired with bq40z50 (Fuel Gauge).

•

Exact Dimensions (PMIC Carrier Board): 50.0 mm  40.0 mm  8.0 mm (integrated into the Power Brick module).

•

Mounting: Screw-secured inside the Power Brick housing via four M2 standoffs.

•

Thermal & Service Envelope: Requires direct thermal contact via a 1.5 mm phase-change pad against the internal aluminum chassis wall for heat dissipation during 15V/3A USB-C fast charging.

8. UWB Generation & Transceiver IC

•

Component Model: Qorvo DWM3000 Module (IEEE 802.15.4z Ultra-Wideband Transceiver based on DW3000 IC).

•

Exact Dimensions: 23.0 mm  13.0 mm  2.4 mm surface-mount module.

•

Antenna Integration: On-board UWB chip antenna requiring a strict 5.0 mm metal-free clearance zone on all sides to prevent multipath signal reflection and antenna detuning.

•

Mounting & Envelope: Positioned on the Sensor Brick perimeter closest to exterior non-conductive plastic windows to maximize line-of-sight ranging accuracy.